Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE734 |
| Course Title | Low Power Digital Integrated Circuits |
| Semester/Year | F2025 |
| Instructor | Dr. Andy Ye |

| Lab/Tutorial Report NO. | 1 |
| --- | --- |

| Report Title | Lab 1: Characteristics of MOSFET Devices |
| --- | --- |

| Section No. | 02 |
| --- | --- |
| Submission Date | October 16, 2025 |
| Due Date | October 16, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Objectives	2

Procedure	2

Post-Layout Simulation	2

Results	2

Discussion	9

Conclusion	10

References	10

## Objectives

This lab investigates the physical characteristics of both NMOS and PMOS transistors, including I-V characteristics, subthreshold conduction, body effect, and intrinsic device capacitances. The lab also creates transistor layouts, running DRC and LVS checks, executing PEX, as well as compares pre and post-layout simulation results.

## Procedure

Using Cadence, NMOS and PMOS schematics are created from devices in the tsmc18 library. Voltage sources: VD, VG and VB are used with device dimensions W = 1um, and L = 180nm.

Simulations are performed with the voltage sources set to 1.8, 1.8 and 0V respectively. DC analyses are performed and sweeps for VG generated I-V curves for both NMOS and PMOS designs.

The subthreshold conduction was analyzed by sweeping VG. This is completed by setting the Y-axis to a logarithmic scale, and using delta markers to measure the subthreshold slope.

The body effect is analyzed by using the sweep function on VSB from -1.8 to 1.8V and plotting Vth vs VSB.

## Post-Layout Simulation

Using the same testbench, post-layout simulations were performed and results were compared between the schematics and extracted views. Due to parasitics, slight reductions in current and threshold voltage were observed.

## Results

### Schematics

Figure 1. NMOS Transistor Schematic

Figure 2. PMOS Transistor Schematic

### I–V Characteristics

Figure 3. NMOS I–V characteristics showing the relationship between drain current (ID) and drain-to-source voltage (VDS) for different gate voltages (VGS). The curve clearly indicates the transition from the triode to saturation region.

Figure 4. PMOS I–V characteristics showing the relationship between drain current (ID) and drain-to-source voltage (VDS) for different gate voltages (VGS). The current direction and threshold are inverted relative to NMOS behavior.

### Subthreshold Conduction

Figure 5. NMOS subthreshold conduction plot. The linear region on the semi-log scale confirms the exponential relationship between ID and VGS in the subthreshold region.

Figure 6. PMOS subthreshold conduction plot. The subthreshold slope is extracted using delta markers, confirming expected MOSFET subthreshold behavior.

### Body Effect

Figure 7. Threshold voltage (Vth) vs substrate bias (VSB) for NMOS. The plot demonstrates the increase in threshold voltage as substrate bias increases, consistent with the body effect model.

Figure 8. Threshold voltage (Vth) vs substrate bias (VSB) for PMOS. Similar behavior to NMOS is observed, with polarity reversed.

### Layouts

Figure 9. NMOS Transistor Layout View

Figure 10. NMOS Transistor Extracted View

### Post-Layout Comparison

Figure 11. Comparison of schematic-level and post-layout I–V characteristics for NMOS. A slight reduction in current is observed post-layout due to parasitic resistances and capacitances.

Figure 12. Comparison of extracted-level and post-layout I–V characteristics for NMOS. Similar degradation is observed, consistent with parasitic extraction effects.

## Discussion

The I-V graphs confirmed MOSFET saturation and channel length modulation. Subthreshold slope values are accurate and simulate theoretical concepts, demonstrating exponential behavior below Vth. Body effect results are also theoretically sound, showing increases in Vth and VSB. Post-layout simulations show the expected degradation of performance due to parasitics.

Quantitively, the NMOS subthreshold slope was around 90mv/dec, which is very close to the expected value at room temperature. A small deviation of less than 5% is present, however this can be attributed to modeling limitations and process parameters in the PDK. The PMOS subthreshold slope has a similar magnitude to that of the NMOS with an opposite polarity, which is very consistent with theoretical concepts of PMOS. The post-layout I-V graphs show a small reduction in drain current compared to the schematic level simulations. This is primarily due to having parasitic resistances and capacitances when creating the layout. The extracted parasitics behave exactly as expected and slightly degrade performance, and results match Cadence design with theory.

## Conclusion

This lab provides real-world experience in terms of MOSFET characterization, designs, layouts, verifications, parasitic extractions, as well as post-layout analyses. The captured results align with theory, and demonstrate the importance of parasitic elements on circuit behavior.

## References

References

Toronto Metropolitan University. (2024, April 15). Lab 1 Video - Part 2 [Video]. D2L. https://courses.torontomu.ca/d2l/le/content/1086714/viewContent/6628118/View

Toronto Metropolitan University. (2024, April 15). Lab 1 Video - Part 1 [Video]. D2L. https://courses.torontomu.ca/d2l/le/content/1086714/viewContent/6628117/View

Toronto Metropolitan University. (2024, April 15). Lab 1 Video - Part 2 (Updated) [Video]. D2L. https://courses.torontomu.ca/d2l/le/content/1086714/viewContent/6628119/View

Toronto Metropolitan University. (n.d.). Lab 1: Characteristics of MOSFET Devices [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/1086714/viewContent/6628060/View