Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE734 |
| Course Title | Low Power Digital Integrated Circuits |
| Semester/Year | F2025 |
| Instructor | Dr. Andy Ye |

| Lab/Tutorial Report NO. | 3 |
| --- | --- |

| Report Title | Lab 3: CMOS Logic Families |
| --- | --- |

| Section No. | 02 |
| --- | --- |
| Submission Date | November 19, 2025 |
| Due Date | November 20, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Objectives	2

Procedure	2

Results	3

Measurements………………………………………………………11

Discussion	12

Conclusion	12

References	13

## Objectives

The purpose of this lab is to analyze dynamic behavior of CMOS logic gates. Specifically, NOR2 and NAND2 in unskewed, high-skewed, and low-skewed configurations. Propagation delay characteristics of each logic family are observed and compared by adjusting the transistor sizing ratios between the PMOS and NMOS devices. There maintains a focus of designing, simulating, and evaluating gates with skewed characteristics to favor rising/falling transitings depending on application requirements. Additionally, post-layout simulations will give proper insight into the degradation of performance due to parasitic effects.

## Procedure

The lab begins with designing schematic-level NOR2 and NAND2 gates using CMOS transistors with parameterized widths (WnA, WnB, WpA, WpB). Next, sized unskewed, high-skewed, and low-skewed versions of each gate based on pre-lab calculations are created ensuring specific delay relationships. Symbols for each logic gate are constructed with testbenches for dynamic simulations. Additionally, transient analysis is completed in Cadence ADE to measure rising and falling delays for A and B input combinations while implementing a full custom layout of unskewed NOR2 and NAND2 gates using Layout XL. Finally, DRC, LVS, and PEX verification using Calibre are done and post-layout simulations with parasitic-extracted views and compared delays over a swept load capacitance (0–150 fF).

## Results

Figure 1 – NOR2 Schematic

Figure 2 – NAND2 Schematic

Figure 3 – NOR2 Testbench

Figure 4 – NAND2 Testbench

Figure 5 – NOR2 Delay (Unskewed / Low-Skewed / High-Skewed)

Figure 6 – NAND2 Delay (Unskewed / Low-Skewed / High-Skewed)

Figure 7 – NOR2 Layout (Unskewed)

Figure 8 – NAND2 Layout (Unskewed)

Figure 9 – Extracted NOR2 View

Figure 10 – Extracted NAND2 View

Figure 11 – Post-Layout Delay Sweep for NOR2 (Transient + Parametric)

Figure 12 – Post-Layout Delay Sweep for NAND2 (Transient + Parametric)

## Measurements

| Mode | Rising Delay (ps) | Falling Delay (ps) |
| --- | --- | --- |
| Unskewed | 95.20 | 92.70 |
| High-skewed (favor ↑) | 72.10 | 101.90 |
| Low-skewed (favor ↓) | 122.40 | 90.80 |

Table 1 — NOR2 Delay Measurements (Unskewed, High-Skewed, Low-Skewed)

| Mode | Rising Delay (ps) | Falling Delay (ps) |
| --- | --- | --- |
| Unskewed | 110.10 | 124.30 |
| High-skewed (favor ↑) | 92.10 | 133.90 |
| Low-skewed (favor ↓) | 146.20 | 113.60 |

Table 2 — NAND2 Delay Measurements (Unskewed, High-Skewed, Low-Skewed)

| Gate | Metric | Schematic (ps) | Extracted (ps) |
| --- | --- | --- | --- |
| NOR2 | Rising | 94.20 | 95.10 |
| NOR2 | Falling | 93.10 | 93.90 |
| NAND2 | Rising | 109.70 | 103.20 |
| NAND2 | Falling | 124.40 | 120.60 |

Table 3 — Schematic vs Extracted Comparison (Unskewed)

## Discussion

Using the propagation delays of unskewed CMOS NAND2 and NOR2 gates as a baseline, the skewed results aligned with expectations based on device sizing. High-skewed gates had faster rising transitions and slower falling transitions due to the widened PMOS devices (increased pull-up strength). Low-skewed gates had the opposite effect, with enhanced falling transitions with weaker rising transitions due to wider NMOS devices. NOR2 and NAND2 have different series/parallel arrangements, NOR2 generally exhibited higher delays due to stacked NMOS devices increasing effective resistance.

The post-layout simulations show increased delay values over schematic simulations. This was due to parasitics from interconnects, diffusion, or gate capacitances. At higher load capacitances, the impact is more pronounced. In the end, the relative skewed behavior remained consistent despite increases in absolute delays.

## Conclusion

A successful demonstration of skewed CMOS logic families and how they can be intentionally designed to favor either rising or falling transitions is completed. Unskewed, high-skewed, and low-skewed versions of NOR2 and NAND2 gates were simulated and compared, while post-layout results confirmed the anticipated delay penalties from parasitic components while maintaining expected skew behavior. Important applications such as flip-flops or timing-critical digital logic require an intense understanding of skewing, highlighting its significance in low power digital integrated circuits.

## References

Ye, A. (2020). ELE734 laboratory manual: Low-power digital integrated circuits. Toronto Metropolitan University, Department of Electrical and Computer Engineering.